1964 Corvair Stock Reference Manual

Notes:

This is a first draft.  I’m sure there will be plenty of mistakes.  I welcome all suggestions and corrections.

These notes are based on my knowledge and inspection of 1964 Monza Sedans.  As such, they will be incomplete as regards other models and trim levels.

I am following the general format of the 3rd edition 1963-64 NCRS Corvette Technical Information Manual and judging guide.  We do not need to use this format in our final document, but I find it a useful format to organize my information gathering.

When I am very uncertain about my information, I have put it in Bold underlined Italics.

We should include a table of abbreviations to make typing easier.  For instance:

SS = Stainless Steel

Zn = Zinc plated

Cd = Cadmium plated

Cr = Chrome plated

Zn/Cd = Zinc or Cadmium Plated

Ohps = Oval head Philips Screw

Ohss = Oval Head Slotted Screw

Phps = Pan Head Phillips Screw

Fhps = Flat Head Phillips Screw

R/L = Right/Left

Thank you,

William Hubbell

# Interior Section

## Door Panel Area

### Door Panels

#### 500 Models:

Two-color vinyl panel

#### 700 Models:

Two-color panels with horizontal mylar strip and mylar “700” above the arm rests.

#### 600/900 Models (Monza):

Panels are made of rigid paperboard with metal edging covered with vinyl.  The vinyl is leather-grained and color-keyed to the interior.  There is perimeter stitching about ½” from the edge of the panel.  The horizontally pleated pattern of the door panel matches the seat vinyl pattern.  The pleats are only found on the upper half of the vinyl (but not in the armrest section). An embossed mylar Monza emblem is located above the armrests.  (Original emblems have sharp grain detail within the emblem; reproductions are smoother and lack the grain.)  A stainless steel (SS) metal strip about ½” wide, and the full length of the door, divides the upper and lower halves of the panel.  Map pockets are located on the lower front half of the front doors.  Rear door panels (on Sedans) are similar to front door panels, but lacking the map pockets. ¼ Window panels (on Coupes) are similar, but have a built-in ashtray.

#### All Models:

Door panels are attached to doors with “Nail Strips” which edge the paperboard and are hidden by the vinyl.  No screws or other retainers are visible along the edge of the panels.  No carpeting is present on the panels.  Panels should lie flat against the door, not warped.  Vinyl should be tightly attached.

### Armrests

#### 500 Models:

Color-keyed plastic base with vinyl covered foam pad.
Front armrests were standard.
Optional rear armrests were of the same basic design as fronts, but incorporated a SS ashtray
Size and shape of both front and rear armrests (and ashtray) are different than Monza type.

#### 700 Models:

Same as 500 Models (?)

#### 600/900 Models (Monza):

Vacuum Plated (“Chromed plastic”) base secured to the door with 2 large Zn/Cd  Phps.
Rear door armrests have an additional smaller head Zn/Cd Phps  at the front.
Armrest pad is vinyl-covered foam pad, secured to the base with 2 Zn/Cd Fhps.
Vinyl is color-keyed to the interior and should neatly attach to the base with a minimum of folds or pulls.
Front armrest pad extends the entire length of the base.  Front armrests are interchangeable right and left
Rear armrests (sedans only) come in right/left versions.  The pad covers the rear 2/3rds of the base and a chromed ashtray with spring loaded lid fits in the front 1/3rd.

### Window Cranks

#### 500/700 Models:

Solid Handle Style

#### 900 Models:

Chrome plated potmetal with a slotted handle and a black “Bakelite” plastic knob, attached to the regulator shaft via a hidden “C” clip.  Both the center of the handle and the knob have a concave concentric swirl pattern.  The knob is riveted to the handle and cannot be removed.   A gray or off-white plastic washer the same diameter as the center of the crank is present between the crank and the door panel.

Right and left window cranks should be installed in symmetrical positions.  Correct installation of the window crank positions the knob toward the front of the car with the window fully up, so as to avoid interference with the door release handle.

### Door Handles

#### 500/700 Models:

Solid Handle Style

#### 600/900 Models:

Chrome plated potmetal with a slotted handle that curves inward at the tip.  Center of the handle has the same concave concentric swirl pattern as the window crank.  Handles are attached in the same manner as the window cranks, with the same plastic washer arrangement.

Right and left door handles should be installed in symmetrical positions.  Correct placement of the door handle is with the handle pointing down and forwards (approximately 4 o’clock on left, 7 o’clock on right).

### Locks

All models the same:

Front doors have no lock button.  Door handle is pushed forward to lock front door.
Rear door lock buttons are plastic color-keyed to interior, with knob having a flared tip with concave concentric swirl pattern, similar to window crank and door handles.  There are no clear bezel rings around the lock knobs (as there are in late models).

### Door Frame

The remainder of the door interior (that part not covered by the door panel) is painted metal.  The color is indicated by the Trim code on the Body plate in the engine compartment, and matches the color of the roof pillars.  This is approximately a 60% gloss finish.

### Vent Window

Triangular-shaped glass pressed into a chromed frame (top and bottom only; rear edge of glass is free).

Vent Window handle are “L”-shaped chromed pot metal, secured to vent window frame post with a stainless pin.  There is a wavy SS washer spring between the handle and the frame.  The rear portion of the vent window frame also contains the front channel of the door window.  The front window channel liner has stainless steel beading, and is lined with black “fuzzy” material.  The inner and outer sides of the rear vent frame are trimmed in stainless steel.

### Options

Mirror remote control for driver’s door, located in the metal part of the forward edge of door, just above the window crank.  It is a chrome diamond-shaped plate secured with two chrome oval head Phillips screws.  A black plastic knob with a ball/socket joint is mounted in the plate.

## Door Opening Area

### Body Jambs

Jambs are painted exterior body color.  Condition of finish may not be even or highly polished.
The VIN plate (Stainless Steel) is spot welded to the rear jamb of the Driver’s door.

### Pinch Welting (Windlace)

The entire jamb pinch weld is covered from front end of door sill around perimeter of door opening to rear end of door sill with vinyl pinch welting (also called windlace).  The pinch welting is unbroken in its length and makes a 90-degree bend at the upper rear corner of the front door (and upper front corner of rear door).  The ends should be tucked under the door sill moldings, leaving no part of the pinch weld visible.  The pinch welting is color-keyed to the interior and has an embossed weave pattern.

### Door Sills

Door sills are made of stamped full tempered hard aluminum alloy that is polished before anodizing. The anodizing used  is known as R5 or 'bright dip' anodizing.  They are smooth on their outer surfaces, while the inner surfaces are embossed with fine lines in 4” sections separated by 1¼” smooth spaces.  A “Body by Fisher” Coach emblem is riveted in the center of the front door sill plate; rear door sill plates lack this emblem.

#### Coupes:

Each front sill plate is secured with 7-chromed oval head Phillips screws (4 on top, 3 on inner side).

The front sill plate overlaps the front “cardboard” kick panel, and interfaces with the front heater grill.

The front sill plate overlaps the rear sill plate.  The rear sill is just a smooth curved piece secured with two screws on its inner surface.

#### Sedans:

Each front and rear sill plates is secured with 5 chromed oval head Phillips screws (3 on top, 2 on inner side).  The front sill plate overlaps the front “cardboard” kick panel, and interfaces with the front heater grill.  The rear sill plate overlaps the front sill plate.

### Strikers

Strikers are located on the rear jambs and are unpainted, natural diecast metal.  They are secured with 3 large flathead Phillips screws with starred lockwashers.  Occasionally, there will be one or more spacers located between the striker and the jamb.  Front and rear strikers are identical; left and right are mirror images.

### Switches

500: none

600/700/900: Front door light switches are single Zn/Cd push-button metal plunger in a 7/16” hex nut base, located on the forward jamb, about 2” below the top hinge.
Rear doors have no switches.

### Hinges

Hinges are painted exterior body color, same as jambs.  Paint may be irregular; some grease is expected.
The front door lower hinge has a spring-loaded hold-open mechanism.
All hinges are attached to the doors with (3) large flathead Phillips screws with starred lock washer.
The front door hinges are each secured to the body via (3) hex bolts from inside the jamb; only the threads are visible.

The rear lower hinge is attached to the body with the same (3) Flathead Phillips screws as above.

The rear upper hinge is attached to the body with (3) ½” hex head Phillips bolts.

Caulking compound may be visible under the hinges.

### Door Jambs

The door weather-stripping separates the color of the jamb: body color on the outside, interior color on the inside.  Color may be uneven on the bottom of the door.

Rear doors have a chromed 7/16” hex head bolt with large flat washer on the rear surface, midway between door bumpers.  (Adjustment for rear window channel)

### Door Latch

Door latch is located on rear surface of door jamb, secured with 3 Zn/Cd oval-head Phillips screws.  Latch is Zn/Cd plated, usually some grease is present.

Front/Rear latches identical; Right/Left are mirror images.

### Door Bumpers

Coupes: Two Rectangular rubber bumpers on rear jamb

Sedans: One Rectangular rubber bumper on rear jamb (front/rear doors); Rear door has additional round bumper on lower end of rear jamb.

Convertibles: Same as Coupe, but with additional wedges

### Weather-strip

Black, pre-molded rubber secured to door with cement (top and front edge) and pins.  Stock weather-strip has large pins; reproduction weather-strip uses smaller pins combined with filler plugs inserted in door.
Weather-strip is seamed at the upper rear corner of front door; upper front corner of rear doors.
No overspray should be evident on weather-strip.

## Interior Roof Area

### Headliner

.

| Model | Material | Colors Available |
| --- | --- | --- |
| 500 | Vinyl ?? | Aqua, Fawn, Ivory |
| 700 | Vinyl ?? | Aqua, Fawn, Ivory, Blue |
| 600, 900 | Vinyl (Waffle Pattern) | Aqua, Black, Blue, Fawn, Ivory, Saddle |

Headliner is color-keyed to the interior.  Red interior uses ivory headliner

Coupes:

Sedans:

Six panels stitched together (five seams).  2nd seam (from front) lines up with rear edge of front door pinchwelting.  Should be smooth with no sags or wrinkles.

Front and rear edges are finished with flat steel trim with rolled edges, painted interior color (same as doors), secured with Cr Ohps.  Both front and rear trim in left and right sections.  Front trim sections joined by overlapping mirror support.  Right rear trim overlaps left rear at center.

Side edges finished with pinchwelting over doors and with “Fat T” shaped painted metal trim between doors.  Trim secured with single Cr Ohps.  R/L trim interchangeable.

### Dome Light

Application: All models except Convertible

Base: Teardrop shaped plastic.  500: Grey.   All others: ”Chromed.

Lens: White translucent plastic

Position: Centered L/R

Coupe – Wide end points forward

Sedan – Wide end points rearward.  Centered between 2nd and 3rd headliner seams.

### Coat Hooks

Application: All models except Convertible

Stamped Cr steel, wide base, narrow point.  Secured with single Cr Ohps.

Location:

Coupe:

Sedan: over rear windows, edge of headliner, centered between 3rd and 4th seams.

### Sunvisors

Application: All models

Covering (color-matched to interior):

500/700: Cardboard?

600/900 Coupe/Sedan: Headliner matching material, edged with smoother version of headliner vinyl Ends of edging finished with Cr clips.

Convertible: Smooth vinyl

Support:

500/700: Silver painted, part of sunvisor

600/900: Cr potmetal three-legged base with spring loaded arm.

Arm has single Cr Ohss to clamp onto sunvisor pivot post

Support attached to roof with 3 Cr Ohps

### Rear View Mirror

Application: All years and models

Size: 8”x2” (all)

Housing: 500/700: Silver-painted.  600/900: SS

Standard: No day/night feature (all)

Attachment: 3 Cr Ohps (arm) 1 Cr Ohss (mirror)

Arm: 500/700: Silver paint.  600/900: Chromed

Day/Night Feature: Optional– black square bakelite knob on bottom, rotates 90 degrees to operate.

Markings (day/night): “Guide Glare-proof” stamped on top

Convertible mirrors are different and have provision for sunvisor support.

### Roof Pillars

Coupe: Front are painted interior color.  Rear are trimmed with Headliner material

Sedan: All are painted interior color.

## Windshield Trim Area

### Front Windshield

Top: See Headliner

Sides: See Roof Pillars

Bottom: See Dash

### Rear Windshield

Top: See Headliner

Sides: See Roof Pillars

Bottom: See Rear Storage Area

## Dash Panel Area

### Dash Pad/Paint

Colors: See Trim Code Section

Paint is flat (0% gloss)

Speaker opening (all cars): 4 rows of 28 vertical slots, center of dash

Ash Tray (all cars) Center of dash, between radio grill and front edge of dash., has a lid which flips open from bottom up.

Pads were optional for all models (see option codes), dash colored vinyl vacuum formed over foam padding.  There is a dash colored rectangular metal trim ring around the speaker opening.

Front edge is trimmed with 4 pieces flat steel trim with rolled edges, painted dash color, secured with Cr Ohps

### Instruments

### Center Area

### Glove Compartment Area

### Controls

## Steering

## Shifter

## Seating Area

## Rear Storage Area

### Trim Panel (Cardboard)

### Vinyl Trim

### Folding Rear Seatback

## Carpet Area

## Under Dash Area

## Overall Cleanliness